NIELIT

# LAB Workshop on 'FPGA Architecture and Programming using Verilog HDL' Batch 2

MINIPROJECT

Student: Chandani Lapasia

User id: fpga0522-chan50

Contents:

- Design Code Snippet

- Testbench Code Snippet

- Simulation Output

- Schematic Diagram

- Synthesis Report

- Output on FPGA Kit

Design Code Snippet:

module top(i3,i2,i1,i0,clk,reset,Y,data);

input i3, i2, i1, i0, clk, reset;

output [3:0] Y;

output data;

wire [3:0] temp1;

wire temp2,temp3,temp4;

ila_0 my_ila_new (.clk(clk),.probe0({temp2,temp3}));

my_clk_div u_my_clk_div(.clk(clk),.clk_div(temp4));

my_latch u_latch(.I(temp1), .clk(clk),.reset(reset),.Y(Y));

my_decoder u_decoder(.i0(temp3),.i1(temp2),.en(data), .y(temp1));

my_mux u_mux(.data(data), .i3(i3),.i2(i2),.i1(i1),.i0(i0),.s1(temp2),.s0(temp3));

my_counter u_counter(.clk(temp4),.s1(temp2),.s0(temp3));

endmodule

module my_latch(I, clk,reset,Y);

input [3:0] I;

input clk,reset;

output [3:0] Y;

reg [3:0] Y;

always @( clk or I or reset)

begin

if(!reset)

Y<=4'b0000;

else

Y<=I;

end

endmodule

module my_decoder(i0,i1,en,y);

input i0,i1,en;

output [3:0] y;

reg [3:0] y;

always @(en or i0 or i1)

begin

if(en)

begin

case({i1,i0})

2'b00: y=4'b1000;

2'b01:y=4'b0100;

2'b10:y=4'b0010;

2'b11:y=4'b0001;

endcase

end

else

y=4'b0000;

end

endmodule

module my_clk_div(clk,clk_div);

input clk;

output reg clk_div;

reg [25:0] temp=26'd0;

always @(posedge clk)

begin

temp<=temp+26'd1;

clk_div<=temp[20];

end

endmodule

module my_counter(clk,s0,s1);

input clk;

output s0,s1;

c_counter_binary_0 u6 ( .CLK(clk),   .Q({s1,s0}) );

endmodule

module my_mux(data, i3,i2,i1,i0,s1,s0);

input i3,i2,i1,i0,s0,s1;

output data;

assign data = (i0 && ~s1 && ~s0) || (i1 && ~s1 && s0) || (i2 && s1 && ~s0) || (i3 && s1 && s0);

endmodule

Testbench Code Snippet:

module tb();

reg i3,i2,i1,i0,clk,reset;

wire [3:0] Y;

wire data;

top u0_top(i3,i2,i1,i0,clk,reset,Y,data);

initial begin

reset=1'b0;clk=0; i3=1;i2=0;i1=1;i0=0;

#10 reset= 1'b1;i3=0;i2=0;i1=0;i0=1;

#10 reset= 1'b1;i3=1;i2=1;i1=1;i0=1;

end

always forever #5 clk = ~clk;

initial begin

$monitor ($time, "\t", "i3=%d i2=%d i1=%d i0=%d Y=%d ", i3,i2,i1,i0,Y);

end

endmodule

Simulation Output :

Schematic Report:

Synthesis Report:

Start Writing Synthesis Report

---------------------------------------------------------------------------------

Report BlackBoxes:

+------+-------------------+----------+

|      |BlackBox name      |Instances |

+------+-------------------+----------+

|1     |c_counter_binary_0 |         1|

|2     |ila_0              |         1|

+------+-------------------+----------+

Report Cell Usage:

+------+-----------------+------+

|      |Cell             |Count |

+------+-----------------+------+

|1     |c_counter_binary |     1|

|2     |ila              |     1|

|3     |BUFG             |     1|

|4     |CARRY4           |     6|

|5     |LUT1             |     1|

|6     |LUT2             |     4|

|7     |LUT3             |     4|

|8     |LUT6             |     1|

|9     |FDRE             |    22|

|10    |IBUF             |     6|

|11    |OBUF             |     5|

+------+-----------------+------+

---------------------------------------------------------------------------------

Finished Writing Synthesis Report : Time (s): cpu = 00:00:25 ; elapsed = 00:00:26 . Memory (MB): peak = 1132.832 ; gain = 112.043

---------------------------------------------------------------------------------

Synthesis finished with 0 errors, 0 critical warnings and 0 warnings.

Output Waveform: